SECTION 10 11 00

VISUAL DISPLAY BOARDS

PART 1- GENERAL

1.1		SUMMARY

A.	Section Includes:

1.	INTERSECTION Floor-to Ceiling Fixed Divider System and Materials

a. Glass - Markerboard, Decorative, Laminated

b. Acousticor (PET) - Solid or Patterned

c. CPS Porcelain Steel Marker Board

d. Notice Board - Fabric Tackable

e. Wood, Steel, Aluminum, Plastic Laminate – Solid or Patterned

B.	Related Sections:

1.	Section 06 10 00 – Rough Carpentry: Wood blocking

2.	Section 09 29 00 – Gypsum Board

3.	Section 09 72 00 – Wall Coverings

4.	Section 09 91 00 – Painting

1.2		SUBMITTALS

A.	Product Data: Provide technical data for products specified.

B.	Shop Drawings: Provide shop drawings for each type of visual display board specified, including section details indicating trim, face, core and backing materials, wall elevations, dimensions, joint locations between panels exceeding maximum panel length and anchor and installation details

C.	Product Samples: Submit manufacturer’s color chart showing the full range of colors available for the following:

1.	Acousticor Solid Panels and Available Cut-out Patterns: Actual finish samples and digital representation of all patterns available

2.	Aluminum Trim and Accessories: Samples of each type and color, on 4-inch (100mm) long sections.  Include sample sets showing the full range of color variations expected

D.	Contract Closeout Submittals:

1.	Maintenance Data: Manufacturer’s cleaning and maintenance instructions covering both routine (daily or weekly) and long term (yearly or longer) operations

2.	Warranty: Executed copy of manufacturer’s warranty

1.3		DELIVERY, STORAGE AND HANDLING

A.	Storage and Protection:

1.	Store factory framed units vertically with packaging materials between each unit to prevent damage

2.	Store materials in dry areas at temperatures above 55º F

1.4		PROJECT CONDITIONS

A.	Field Measurements: Verify field measurements before fabrication to ensure proper fitting.  Coordinate fabrication schedule with construction progress to avoid delaying the work. Notify the Architect of any conflicts with other construction such as casework, electrical switches, outlets, clocks, mechanical openings, fire detector devices, etc

B.	Environmental Requirements: Install only when the building is enclosed, and interior air and substrate temperatures are stable and approximate the required design conditions

1.5		WARRANTY

- ACOUSTICOR (PET) – Provide warranty on product against color fading and ASTM E84 CLASS A FIRE RATING degradation

- Warranty Period: 1 year on ACOUSTICOR 100% Polyester Acoustic Felt Panels

PART 2- PRODUCTS

2.1		MANUFACTURERS

A.	Basis-of-Design Product: Corona Group INTERSECTION FIXED Floor-to-Ceiling Divider System:

Tel. 205.941.1942, info@coronagroupinc.com

B.	Approved Alternates: Due to the unique nature of this product there are no known alternates

C.	Additional alternates must have prior approval.  Submit technical information and samples to architect a minimum of 14 days prior to bid date

2.2	INTERSECTION FLOOR-TO-CEILING FIXED DIVIDER SYSTEM

A.	INTERSECTION Fixed Divider System

- Standard 1.5” Square Tubes Finished on all sides, with integral T-slots only where required to create unitized panels or starter/add-on panels

- Satin Anodized or Powder Coated Extruded Aluminum, 6105-T5 Alloy with T-slots as designated in drawings

- 2.3		FRAMELESS ACOUSTICOR PANELS

- Frameless PET Panels:  Tackable, acoustical panels.

- Finished Surface: Integrally colored PET Felt Panels, max 4’ x 10’, 3/8” (9mm), self-healing tackable surface made of 100% Polyester Fiber Felt (60% recycled and 40% Virgin).  ASTM E84 CLASS A Fire Rating

- 2.4		FRAMED LAMINATED ACOUSTICOR PANELS WITH SIDE B OR 		INTERLAYER

- Framed PET Panels:  Laminated tackable, acoustical panels

- Finished Surface: Integrally colored PET Felt Panels, max 4’ x 10’, 3/8” (9mm), self-healing tackable surface made of 100% Polyester Fiber Felt (60% recycled and 40% Virgin).  ASTM E84 CLASS A Fire Rating

- Cut-out Pattern according to Corona Standards or Custom as Specified

- Laminated to a Side B material or with a background interlayer for graphic cut-out

2.5		FABRICATION

A.	According to Signed Authorized Shop Drawings

B.	Assembly: Provide factory-assembled divider units, as required.  Provide all Posts and Rails and Levelers to make unitized or starter/add-on panels and deliver to the job site for installation using concealed fasteners according to the ceiling type

2.6		FINISHES

- Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for recommendations relative to applying and designating finishes

B	Manufacturer’s Standard finishes as designated in the finish schedule

PART 3- EXECUTION

3.1		EXAMINATION

A.	Examine FLOOR AND CEILING surfaces, with installer present, for compliance with requirements and other conditions affecting divider installation

B.	Verify flat floor and ceiling surfaces for proper board installation; provide concealed shims as necessary within Architect’s acceptable variance.

C.	Do not proceed with installation until unsatisfactory conditions have been corrected

3.2		INSTALLATION

- Deliver factory-built divider screens and frames, completely assembled in one piece without joints, where possible

- Install units in locations and mounting heights as indicated on drawings and according to manufacturer’s written instructions.  Keep perimeter lines straight, plumb and level. Provide grounds, clips, brackets, anchors, trim and accessories necessary for complete installation

3.3		CLEANING

A.	Clean units according to manufacturer’s written instructions

B.	Cover surface of all units with protective cover taped to frame

C.	Remove protective covers and tape on date of Substantial Completion

END OF SECTION

Rev. 06-12-2023