SECTION 10 11 00

VISUAL DISPLAY BOARDS

PART 1 - GENERAL

1.1		SUMMARY

A.	Section Includes:

1.	Manually Operated Vertical Sliding Markerboards (Column Boards)

B.	Related Sections:

1.	Section 06 10 00 – Rough Carpentry: Wood blocking

2.	Section 09 29 00 – Gypsum Board

3.	Section 09 72 00 – Wall Coverings

4.	Section 09 91 00 – Painting

1.2		SUBMITTAL

A.	Product Data: Provide technical data for products specified.

B.	Shop Drawings: Provide shop drawings for each type of visual display board specified, including section details indicating trim, face, core and backing materials, wall elevations, dimensions, joint locations between panels exceeding maximum panel length and anchor and installation details

C.	Product Samples: Submit manufacturer’s color chart showing the full range of colors available for the following:

- Markerboard: Actual sections of porcelain enamel steel finish for each type of chalkboard and whiteboard required

- Aluminum Trim and Accessories: Samples of each type and color, on 6-inch (150mm) long sections of extrusions and not less than 4-inch (100mm) squares of sheet or plate. Include sample sets showing the full range of color variations expected

D.	Contract Closeout Submittals:

1.	Maintenance Data: Manufacturer’s cleaning and maintenance instructions covering both routine (daily or weekly) and long term (yearly or longer) operations

2.   Warranty: Executed copy of manufacturer’s warranty

1.3		DELIVERY, STORAGE AND HANDLING

A.	Storage and Protection:

1.	Store factory framed units vertically with packaging materials between each unit to prevent damage

2.	Store materials in dry areas at temperatures above 55º F

1.4		PROJECT CONDITIONS

A.	Field Measurements: Verify field measurements before fabrication to ensure proper fitting.  Coordinate fabrication schedule with construction progress to avoid delaying the work. Notify the Architect of any conflicts with other construction such as casework, electrical switches, outlets, clocks, mechanical openings, fire detector devices, etc

B.	Environmental Requirements: Install only when the building is enclosed and interior air and substrate temperatures are stable and approximate design conditions

1.5		WARRANTY

A.	Porcelain Enamel Writing Surface Warranty: Submit a written warranty executed by the manufacturer agreeing to replace porcelain enamel steel markerboards that do not retain their original writing and erasing qualities, become slick and shiny (if matt surface), or exhibit crazing, cracking or flaking within the specified warranty period, provided the manufacturer’s written instructions for handling, installation, protection and maintenance have been followed

- Warranty Period: LIFE TIME or a Minimum of (50) years from date of completion on writing surface from delaminating or crazing

B.	Column Board Warranty: Submit a written warranty executed by the manufacturer agreeing to replace Column Boards that exhibit manufacturer defect within the specified warranty period, provided the manufacturer’s written instructions for handling, installation, protection and maintenance have been followed.

1.	Warranty Period: Minimum (2) years from date of completion on mechanical system

PART 2- PRODUCTS

2.1		MANUFACTURERS

A.	Basis-of-Design Product: Vertical Sliding Board (Column Board) based on Corona Group Visual Presentation Products:  Contact:  205.941.1942 info@coronagroupinc.com - www.coronagroupinc.com

B.	Approved Alternate Manufacturers:  Peter Pepper Products, Egan Visual.

C.	Additional alternates must have prior approval.  Submit technical information and samples to architect a minimum of 14 days prior to bid date.

2.2		DUAL COLUMN BOARD UNITS

A.	Corona Column Boards: Vertical Sliding Markerboard Unit consisting of two satin anodized or powder coated aluminum columns per side for the two-board system. Markerboard Surface: Magnetic Porcelain Steel Dry-erase markerboards for marker pens and magnets. Boards run smoothly up and down independently of each other. Square frame and full-length aluminum penshelf. The manually operated sliding boards can slide up and down the full height of the columns. Fixed wall mounted markerboard to be installed behind the column board unit.

1.	Face Sheet: Marker board: CPS Satin Gloss, scratch-resistant                                                               porcelain enamel steel .4 - .6mm thick; fused twice between 1200 and 1600º F

2.	Core: 19mm/25.4mm Substrate of Honeycomb Construction

3.	Backing: Balanced sheet metal backing

4.	Sizes: As indicated in Drawings

5.	Frame Profile: Manufacturer’s VIP FLEX PROFILE

6.	Columns: Two columns required per sliding board unit 4”D X 4”W X 108”H. When specified as a dual unit attachment brackets are included). The system can be field extended with several columns in front and/or continuously side-by-side with each other.

7.	Sliding Board Frames: 6065 Extruded Aluminum, 2mm gauge, 60mmH x 25mmD.

8.	Sliding Tracks (integral): All fittings, wires and counter-balance weights are hidden in the columns and calibrated at factory.

9.	Full width Aluminum Penshelf on each board to serve as a handle allowing smooth operation from any position along the width of the board.

10.	Back Writing Board: Fixed Wall Mounted VIP SERIES Markerboard, to be mounted behind the column board unit - 120” w x 51.3”H with a Dual Track Top Profile including Paper Hanging Ball Bearings.

11.	Accessories: Provide one accessory pack for each Column Board system.  Accessory Pack shall include four (4) dry-erase markers, one (1) magnetic felt eraser, one (1) package of ten (10) replacement eraser pads, one (1) package of three (3) Eximo™ markerboard cleaning pads and one (1) soft drying cloth, twelve (12) round Teflon coated magnets.

12. Board Finish Color: Markerboard – Satin Gloss White.

13. Aluminum Finish:  Satin Anodized or Powder Coated (TBD)

2.4		FABRICATION

- Porcelain Enamel steel writing board facing sheet and backer sheet shall be laminated to the to core material under pressure with manufacturer’s recommended flexible, waterproof adhesive.

- Assembly: Provide factory-assembled Column Board units, as required.  Column Boards up to 12 feet wide should be factory framed in a single unit and delivered to the job site for installation

- Provide manufacturer’s standard Markerboard unit wall mounted behind the column board unit according to plans.

2.5		FINISHES

A.	Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for recommendations relative to applying and designating finishes.

B.     	Manufacturer’s Standard finishes as designated above.

PART 3- EXECUTION

3.1		EXAMINATION

A.	Examine wall surfaces, with installer present, for compliance with requirements and other conditions affecting visual display board/rail installation.

B.	Verify flat wall surfaces for proper board installation; provide concealed shims as necessary within Architect’s acceptable variance. Minimum Level 4 finished wall required by contractor.

C.	Do not proceed with installation until unsatisfactory conditions have been corrected.

3.2		INSTALLATION

A.	Deliver factory built visual display boards/rails, completely assembled in one piece without joints, where possible.

B.	Install units in locations and mounting heights as indicated on drawings and according to manufacturer’s written instructions.  Keep perimeter lines straight, plumb and level. Provide grounds, clips, backing materials, adhesives, brackets, anchors, trim and accessories necessary for complete installation.

C.	Coordinate project site assembled units with grounds, trim and accessories. Join with a neat, precision fit.

3.3		CLEANING

A.	Clean units according to manufacturer’s written instructions.

B.	Cover surface of all units with protective cover taped to frame.

- Remove protective covers and tape on date of Substantial Completion.

END OF DOCUMENT

Rev:  03-09-2021