SECTION 10 11 00

VISUAL DISPLAY BOARDS

PART 1- GENERAL

1.1		SUMMARY

A.	Section Includes:

1.	Magnetic or Non-Magnetic Dry-erase Glass Boards (Frameless)

B.	Related Sections:

1.	Section 06 10 00 – Rough Carpentry: Wood blocking

2.	Section 09 29 00 – Gypsum Board

3.	Section 09 72 00 – Wall Coverings

4.	Section 09 91 00 – Painting

1.2		SUBMITTALS

A.	Product Data: Provide technical data for products specified.

B.	Shop Drawings: Provide shop drawings for each type of visual display board specified, including section details indicating trim, face, core, and backing materials, wall elevations, dimensions, joint locations between panels exceeding maximum panel length and anchor and installation details

C.	Product Samples: Submit manufacturer’s color chart showing the full range of colors available for the following:

1.	Glass board: 12” X 12” Sample to illustrate material, finish, color, and configuration of each type of glass board required

2	Aluminum Trim and Accessories: Samples of each type and color, on 6-inch (150mm) long sections of extrusions and not less than 4-inch (100mm) squares of sheet or plate. Include sample sets showing the full range of color variations expected

D.	Contract Closeout Submittals:

1.	Maintenance Data: Manufacturer’s cleaning and maintenance instructions covering both routine (daily or weekly) and long term (yearly or longer) operations

2.	Warranty: Executed copy of manufacturer’s warranty

1.3		DELIVERY, STORAGE AND HANDLING

A.	Storage and Protection:

1.	Store factory framed or unframed units vertically with packaging materials between each unit to prevent damage

2.	Store materials in dry areas at temperatures above 55º F

1.4		PROJECT CONDITIONS

A.	Field Measurements: Verify field measurements before fabrication to ensure proper fitting.  Coordinate fabrication schedule with construction progress to avoid delaying the work. Notify the Architect of any conflicts with other construction such as casework, electrical switches, outlets, clocks, mechanical openings, fire detector devices, etc.

B.	Environmental Requirements: Install only when the building is enclosed and interior air and substrate temperatures are stable and approximate design conditions

1.5		WARRANTY

A.	Glass Writing Surface Warranty: Submit a written warranty executed by the manufacturer agreeing to replace glass boards that discolor due to cleaning, staining or use.

1.	Warranty Period: Minimum (10) years from date of completion

PART 2- PRODUCTS

2.1		MANUFACTURERS

A.	Basis-of-Design Product: Corona Group Frameless Dry-erase Starphire Low Iron Glass Marker Boards Contact info@coronagroupinc.com or call coronaVisual @ 205-941-1942 – coronagroupinc.com

B.	Additional alternates must have prior approval.  Submit technical information and samples to architect a minimum of 14 days prior to bid date

2.2	MAGNETIC or NON-MAGNETIC (PLEASE SELECT ONE) STARPHIRE LOW IRON GLASS BOARDS – FRAMELESS with CONCEALED MOUNTING OPTION OR ON STAND-OFFS (PLEASE SELECT ONE)

A.	Magnetic / Non-Magnetic Glass Marker Boards: Dry-erase Glass Board for marker pens and magnets

1.	Face Sheet: Starphire low-iron, tempered glass with flat polished glass edges –

a.  Thickness: 6mm (1/4”) glass – 1mm galvanized steel plate if Magnetic

b.   Dimensions: __W X __H ( __ sq ft)

2.	Backing: 1mm galvanized steel plate to produce magnetic properties, as well as safety glass properties

3.	Sizes: As indicated in Drawings

4.	Frame Profile: N/A – Frameless Board

5.	Drillings:  If board is mounted on Stand-Offs, specify number of drillings based on the size of the board Minimum drillings is 4(and size of Stand-off if other than standard below).

6.	Mounting Hardware for frameless glass boards:

a.	Concealed Z-clip full width of board minus 4” (2” each side), mounting shall be of a nature that allow the boards to “float” a maximum of 5/8” from face of the wall to face of the glass.  All Boards shall have a minimum of (2) z-clip rows for boards up to 60” H (3) z-clip rows for boards up to 96” High and (4) z-clip rows for Boards over 96”H.  Hardware pack shall include appropriate hollow wall anchors, and rubber bumpers for leveling wall imperfections

b.	Aluminum Stand-off  5/8” x 5/8” (requires 7/16” hole, max thickness of glass ¼”)

c.	Stainless Steel Stand-off  5/8” x 5/8” (requires 7/16” hole, max thickness of glass is ¼”

7.	Accessories:

a.	Provide one accessory pack for each glass board.  Accessory Pack shall include four (4) or (8) colored pens –one (1) magnetic rectangular eraser, with ten (10) eraser refill, Eximo Cleaner Box with (3) Sponges and (1) Cloth, six (6) heavy duty rare earth magnets with protective sleeves (if magnetic), Bottle for Eximo Water, Microfiber Towel and Magnetic Hook (if magnetic)

b.	Optional Penshelf:

1.	Universal Penshelf Aluminum – from 12” up to the full width of glass board – specify: Adhesive Face or Back, Magnetic, or Concealed Wall Mount and length of shelf.

2.	HUB – All-In-One Accessory Station

8.	Board Color: Custom color to be determined and may require custom barrier coat to insure color clarity.

2.3		FABRICATION

A.	Glass Markerboards shall be made using a coating applied to an unexposed surface of glass as an opacifier, resulting in a writable surface. Coating process shall be such that virtually any color option is available and the coating is bonded to the glass to allow direct adhesive with or without the metal plate backing. A 1mm galvanized steel backer shall be laminated to the glass panel to provide magnetic properties. Steel backer to provide full coverage. All edges of a frameless glass board shall be flat polished and cleaned of any overspray.  Glass shall always be ¼” tempered unless otherwise specified.

B.	Assembly: Provide factory-assembled magnetic glass writing board units, as required.  Magnetic glass writing boards shall be delivered in a single panel up to manufacturer’s maximum size to the job site for installation using concealed fasteners. Oversized boards should be installed side by side with 1/8” reveal between each board and installed directly to the wall using concealed fastening systems recommended by the manufacturer.

Joints should be as follows:

1.	Make joints only where total length/height exceeds maximums.  Fabricate minimum number of joints, balanced around center of board, coordinate with Architect for approval	.

2.	Provide manufacturer’s standard vertical butt joint system between abutting sections of writing boards – Standard 1/8” reveal between each board and surrounding millwork for Frameless glass boards.

.

2.4		FINISHES

A.	Custom color tolerance to be within one delta E (CIE LAB @ D65 10°) (ASTM D 2244)

B.	Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for recommendations relative to applying and designating finishes

C.     	Manufacturer’s Standard finishes as designated above

PART 3- EXECUTION

3.1		EXAMINATION

A.	Examine wall surfaces, with installer present, for compliance with requirements and other conditions affecting visual display board/rail installation

B.	Verify flat wall surfaces for proper board installation; provide concealed shims as necessary within Architect’s acceptable variance. Minimum Level 4 finished wall required by contractor.

C.	Do not proceed with installation until unsatisfactory conditions have been corrected

3.2		INSTALLATION

A.	Deliver factory built visual display boards, completely assembled in one piece without joints, where possible.

B.	Install units in locations and mounting heights as indicated on drawings and according to manufacturer’s written instructions.  Keep perimeter lines straight, plumb and level. Provide grounds, clips, backing materials, adhesives, brackets, anchors, trim and accessories necessary for complete installation.

C.	Coordinate project site assembled units with grounds, trim and accessories. Join with a neat, precision fit.

3.3		CLEANING

A.	Clean units according to manufacturer’s written instructions.

B.	Cover surface of all units with protective cover taped to frame.

C.	Remove protective cover and tape on date of Substantial Completion.

END OF SECTION

Rev. 04-22-2021