SECTION 10 11 00

VISUAL DISPLAY BOARDS

PART 1- GENERAL

1.1		SUMMARY

A.	Section Includes:

1.	Dry-erase Marker Boards for INTERSECTION Top-Hung Floor-to-Ceiling Sliding System

2.	Glass Markerboards for Dividers

2.	Steel Mesh Divider Screens

B.	Related Sections:

1.	Section 06 10 00 – Rough Carpentry: Wood blocking

2.	Section 09 29 00 – Gypsum Board

3.	Section 09 72 00 – Wall Coverings

4.	Section 09 91 00 – Painting

1.2		SUBMITTALS

A.	Product Data: Provide technical data for products specified.

B.	Shop Drawings: Provide shop drawings for each type of visual display board specified, including section details indicating trim, face, core and backing materials, wall elevations, dimensions, joint locations between panels exceeding maximum panel length and anchor and installation details

C.	Product Samples: Submit manufacturer’s color chart showing the full range of colors available for the following:

1.	BACK PAINTED GLASS Markerboards: Actual sections of Glass  finish for each type markerboard required

2	Aluminum Trim and Accessories: Samples of each type and color, on 6-inch (150mm) long sections of extrusions and not less than 4-inch (100mm) squares of sheet or plate. Include sample sets showing the full range of color variations expected

D.	Contract Closeout Submittals:

1.	Maintenance Data: Manufacturer’s cleaning and maintenance instructions covering both routine (daily or weekly) and long term (yearly or longer) operations

2.	Warranty: Executed copy of manufacturer’s warranty

1.3		DELIVERY, STORAGE AND HANDLING

A.	Storage and Protection:

1.	Store factory framed units vertically with packaging materials between each unit to prevent damage

2.	Store materials in dry areas at temperatures above 55º F

1.4		PROJECT CONDITIONS

A.	Field Measurements: Verify field measurements before fabrication to ensure proper fitting.  Coordinate fabrication schedule with construction progress to avoid delaying the work. Notify the Architect of any conflicts with other construction such as casework, electrical switches, outlets, clocks, mechanical openings, fire detector devices, etc

B.	Environmental Requirements: Install only when the building is enclosed and interior air and substrate temperatures are stable and approximate design conditions

  - WARRANTY

- Glass Writing Surface Warranty: Submit a written warranty executed by the manufacturer agreeing to replace glass markerboards that do not retain their original writing and erasing qualities, become slick and shiny (if matt surface), or exhibit crazing, cracking or flaking within the specified warranty period, provided the manufacturer’s written instructions for handling, installation, protection and maintenance have been followed

1.	Warranty Period: FOREVER WARRANTY ON GLASS  Markerboards

PART 2- PRODUCTS

2.1		MANUFACTURERS

A.	Basis-of-Design Product: Dry-erase markerboards are based on Corona Group INTERSECTION TOP-HUNG Floor-to-Ceiling Sliding Panel System:

Tel. 205.941.1942, info@coronagroupinc.com

B.	Approved Alternates: Due to the unique nature of this product there are no known alternates.

C.	Additional alternates must have prior approval.  Submit technical information and samples to architect a minimum of 14 days prior to bid date

2.2	MARKERBOARD– INTERSECTION FLOOR-TO-CEILING SLIDING PANEL SYSTEM

A.	INTERSECTION Markerboard: Dry-erase GLASS markerboards for INTERSECTION SLIDE

1.	Face Sheet: ¼” (6MM) TEMPERED GLASS back painted, fitted in T-slots of Intersection Posts and Rails – Color as per Finish Schedule

2.	MESH Screen made of steel, welded joints to form ½” or 1” squares, powder coated Black to match frame

3.	RUBBER GASKET – Fitted rubber gasket for glass and mesh TO fit within the post and rails

B.  	POSTS AND RAILS:  Powder Coated Black Extruded Aluminum, 6105-T5 Alloy with T-slots as designated in drawings.

- ALUMINUM FRAMES:  1.0” Square Tubes finished on all sides and joined with concealed fasteners

- ROLLING GEAR:  Set of Dual Nylon Rollers per Panel complete with Mounting Hardware for attaching Rolling Gear and other Mechanisms to the track system

C.	VERTICAL SLIDING WALL SYSTEM TRACK:  Manufacturer’s standard as per sizes and specifications in Drawings. Track system must be available in UNLIMITED sliding configurations as specified. Powder Coated Black extruded Aluminum, 6105-T5 Alloy, and suitable for hanging and/or sliding various sizes of two-sided materials as described in the drawings.

a.	Top tracks shall be 1 11/32” deep x 1 9/16” high - Powder Coated Black

b.	Bottom tracks consists of three modular parts repeated as per drawings. END: 7/8"w x 5/16"h - BOTTOM: 1"w x 3/16"h - TOP: 1 1/16"w x 1/4"h – Powder Coated Black

2.3		FABRICATION

A.	Single sided ¼” Tempered, back painted Starfire (Low-Iron) Glass writing board

B.	Assembly: Provide factory-assembled writing board units, as required.  Provide all Posts and Rails and Mesh to make unitized panels and deliver to the job site for installation using concealed fasteners.

2.4		FINISHES

A.	Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for recommendations relative to applying and designating finishes

B.     	Manufacturer’s Standard finishes as designated above and on Finish Schedule

PART 3- EXECUTION

3.1		EXAMINATION

A.	Examine FLOOR AND CEILING surfaces, with installer present, for compliance with requirements and other conditions affecting visual display board/track installation

B.	Verify flat floor and ceiling surfaces for proper board installation; provide concealed shims as necessary within Architect’s acceptable variance.

C.	Do not proceed with installation until unsatisfactory conditions have been corrected

3.2		INSTALLATION

- Deliver factory built visual display boards/tracks, completely assembled in one piece without joints, where possible

- Install units in locations and mounting heights as indicated on drawings and according to manufacturer’s written instructions.  Keep perimeter lines straight, plumb and level. Provide grounds, clips, brackets, anchors, trim and accessories necessary for complete installation

- Level and test all sliding components for smooth operation.

3.3		CLEANING

A.	Clean units according to manufacturer’s written instructions

B.	Cover surface of all units with protective cover taped to frame

C.	Remove protective covers and tape on date of Substantial Completion

END OF SECTION

Rev. 04-22-2021