SECTION 10 11 00

VISUAL DISPLAY BOARDS

PART 1- GENERAL

1.1		SUMMARY

A.	Section Includes:

1.	Dry-erase Marker Boards for INTERSECTION STACK Top-Hung Floor-to-Ceiling Center-Pivot Sliding System

2.	CPS Marker Board Panels

3.	ACOUSTICOR Felt Panels

B.	Related Sections:

1.	Section 06 10 00 – Rough Carpentry: Wood blocking

2.	Section 09 29 00 – Gypsum Board

3.	Section 09 72 00 – Wall Coverings

4.	Section 09 91 00 – Painting

1.2		SUBMITTALS

A.	Product Data: Provide technical data for products specified.

B.	Shop Drawings: Provide shop drawings for each type of visual display board specified, including section details indicating trim, face, core and backing materials, wall elevations, dimensions, joint locations between panels exceeding maximum panel length and anchor and installation details

C.	Product Samples: Submit manufacturer’s color chart showing the full range of colors available for the following:

1.	CPS MARKER BOARDS: Actual sections of porcelain enamel steel finish for each type of markerboard required

2.	ACOUSTICOR PET Felt Panels: Chain set of full range of color options in minimum 2” x 4” samples

3.	ALUMINUM TRIM AND ACCESSORIES: Samples of each type and color, on 6-inch (150mm) long sections of extrusions and not less than 4-inch (100mm) squares of sheet or plate. Include sample sets showing the full range of color variations expected

D.	Contract Closeout Submittals:

1.	Maintenance Data: Manufacturer’s cleaning and maintenance instructions covering both routine (daily or weekly) and long term (yearly or longer) operations

2.	Warranty: Executed copy of manufacturer’s warranty

1.3		DELIVERY, STORAGE AND HANDLING

A.	Storage and Protection:

1.	Store factory framed units vertically with packaging materials between each unit to prevent damage

2.	Store materials in dry areas at temperatures above 55º F

1.4		PROJECT CONDITIONS

A.	Field Measurements: Verify field measurements before fabrication to ensure proper fitting.  Coordinate fabrication schedule with construction progress to avoid delaying the work. Notify the Architect of any conflicts with other construction such as casework, electrical switches, outlets, clocks, mechanical openings, fire detector devices, etc

B.	Environmental Requirements: Install only when the building is enclosed and interior air and substrate temperatures are stable and approximate design conditions

1.5		WARRANTY

- Provide warranty on Porcelain Enamel Writing Surface: Submit a written warranty executed by the manufacturer agreeing to replace porcelain enamel steel markerboards/chalkboards that do not retain their original writing and erasing qualities, become slick and shiny (if matt surface), or exhibit crazing, cracking or flaking within the specified warranty period, provided the manufacturer’s written instructions for handling, installation, protection and maintenance have been followed

- Warranty Period: Forever for CPS Porcelain Steel Marker Boards

- Provide warranty on PET product against color fading and ASTM E84 CLASS A FIRE RATING degradation

- Warranty Period: 1 year on ACOUSTICOR 100% Polyester Acoustic Felt Panels

PART 2- PRODUCTS

2.1		MANUFACTURERS

A.	Basis-of-Design Product: Dry-erase markerboards are based on Corona Group INTERSECTION TOP-HUNG Floor-to-Ceiling Center-Pivot Sliding Panel System:

Tel. 205.941.1942, info@coronagroupinc.com

B.	Approved Alternates: Due to the unique nature of this product there are no known alternates.

C.	Additional alternates must have prior approval.  Submit technical information and samples to architect a minimum of 14 days prior to bid date

2.2	MARKERBOARD/ACOUSTICOR – INTERSECTION FLOOR-TO-CEILING CENTER-PIVOT SLIDING DIVIDER SYSTEM

A.	INTERSECTION CPS PORCELAIN STEEL MARKER BOARD

1.  PANELS:  CPS Two-sided Marker Board

a.	Face Sheet: CPS Satin, scratch-resistant, porcelain enamel steel .4-.6mm thick; fused twice between 1200º F - 1600º F to create a NON-POROUS surface

b.	Core:  19mm (3/4”) honeycomb core laminated to CPS Face Sheets on both sides to create a balanced sturdy light weight panel

c.	Top, Bottom and Side Profiles: Satin Anodized or Black Powder Coated extruded aluminum 2mm gauge, 6mm (1/4”) wide x 25mm (1” ) deep

d.	Size: According to drawings and specs

e.	Eco Properties:  99% recyclable, globally Cradle-to-Cradle Certified Bronze

f.	Colors:  Standard is White, Light Grey, Beige available upon request

g.	Patterns:  Simple Grid and Educational patterns available on most boards

B.	INTERSECTION ACOUSTICOR Acoustic Felt - PET

- PANELS:  ACOUSTICOR Felt panels for visual privacy and sound absorption – various sizes per drawing

  - Thickness:  9mm (3/8”) thick

  - Material: 100% Polyester fiber acoustic felt – 60% Recycled PET material and 40% Virgin material

  - Eco Properties: Non-toxic, Non-allergenic, Non-irritant; no coatings/sprays that degrade

  - Acoustic Rating: Average NRC .7 (subject to mounting conditions and application)

  - Fire Rating: ASTM E84 CLASS A

  - Colors:  17 Standard Color Offerings

  - Patterns:  35 standard CNC cut Patterns or according to designer’s specification

- INTERSECTION POSTS AND RAILS

- ALUMINUM FRAMES:  6105-T5 Alloy with T-slots as designated in drawings

  - Size: 1.5” Square Tubes finished on all sides and joined with concealed fasteners

  - Color:  Powder Coated Black Extruded Aluminum

  - Rolling Gear:  Single Axle Centered, Pivoting, Dual Nylon Carriage Roller per Panel complete with Mounting Hardware and machining to the track system – 1 per panel

  - Hinges:  Concealed 2 ¾” Soss Style Hinge – 3 per panel

  - Floor Stop:  Frame Mounted Plunger Style Stop to secure hinged panels in various configurations when in use

- TOP-HUNG SLIDING SYSTEM TRACK

- ALUMINUM SYSTEM:

  - Size:  Top tracks shall be 1 11/32” deep x 1 9/16” high. Track system must be available in various lengths.  See drawings and specs for details

  - Color:  Powder Coated Black extruded Aluminum

  - Material:  6105-T5 Alloy, and suitable for hanging and/or sliding various sizes of two-sided materials as described in the drawings

2.3		FABRICATION

- Provide factory-assembled unitized Divider Screens when feasible for installation on site using concealed fasteners

- On-Site Assembly:  Provide all Posts, Rails and ACOUSTICOR to make unitized panels on site for installation using concealed fasteners

2.4	FINISHES

- Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for recommendations relative to applying and designating finishes

- Manufacturer’s Standard finishes as designated above

PART 3- EXECUTION

3.1		EXAMINATION

A.	Examine FLOOR AND CEILING surfaces, with installer present, for compliance with requirements and other conditions affecting visual display board/track installation

B.	Verify flat floor and ceiling surfaces for proper board installation; provide concealed shims as necessary within Architect’s acceptable variance.

C.	Do not proceed with installation until unsatisfactory conditions have been corrected

3.2		INSTALLATION

- Deliver factory built visual display boards/tracks, completely assembled in one piece without joints, where possible

- Install units in locations and mounting heights as indicated on drawings and according to manufacturer’s written instructions.  Keep perimeter lines straight, plumb and level. Provide grounds, clips, brackets, anchors, trim and accessories necessary for complete installation

- Level and test all sliding components for smooth operation.

3.3		CLEANING

A.	Clean units according to manufacturer’s written instructions

B.	Cover surface of all units with protective cover taped to frame

C.	Remove protective covers and tape on date of Substantial Completion

END OF SECTION

Rev. 04-22-2021