SECTION 10 11 00

VISUAL DISPLAY BOARDS

PART 1- GENERAL

1.1		SUMMARY

A.	Section Includes:

1.	INTERSECTION WEAVE Floor-to-Ceiling Fixed Divider System

2.	ACOUSTICOR Weave Divider Slats

B.	Related Sections:

1.	Section 06 10 00 – Rough Carpentry: Wood blocking

2.	Section 09 29 00 – Gypsum Board

3.	Section 09 72 00 – Wall Coverings

4.	Section 09 91 00 – Painting

1.2		SUBMITTALS

A.	Product Data: Provide technical data for products specified.

B.	Shop Drawings: Provide shop drawings for each type of visual display board specified, including section details indicating trim, face, core and backing materials, wall elevations, dimensions, joint locations between panels exceeding maximum panel length and anchor and installation details

C.	Product Samples: Submit manufacturer’s color chart showing the full range of colors available for the following:

1.	ACOUSTICOR PET Felt Panels: Chain set of full range of color options in minimum 2” x 4” samples

2	Aluminum Trim and Accessories: Samples of each type and color, on 6-inch (150mm) long sections.  Include sample sets showing the full range of color variations expected

D.	Contract Closeout Submittals:

1.	Maintenance Data: Manufacturer’s cleaning and maintenance instructions covering both routine (daily or weekly) and long term (yearly or longer) operations

2.	Warranty: Executed copy of manufacturer’s warranty

1.3		DELIVERY, STORAGE AND HANDLING

A.	Storage and Protection:

1.	Store factory framed units vertically with packaging materials between each unit to prevent damage

2.	Store materials in dry areas at temperatures above 55º F

1.4		PROJECT CONDITIONS

A.	Field Measurements: Verify field measurements before fabrication to ensure proper fitting.  Coordinate fabrication schedule with construction progress to avoid delaying the work. Notify the Architect of any conflicts with other construction such as casework, electrical switches, outlets, clocks, mechanical openings, fire detector devices, etc

B.	Environmental Requirements: Install only when the building is enclosed and interior air and substrate temperatures are stable and approximate design conditions

1.5		WARRANTY

- Provide warranty on product against color fading and ASTM E84 CLASS A FIRE RATING degradation

- Warranty Period: 1 year on ACOUSTICOR 100% Polyester Acoustic Felt Panels

PART 2- PRODUCTS

2.1		MANUFACTURERS

A.	Basis-of-Design Product: Acoustic Weave Dividers are based on Corona Group INTERSECTION WEAVE FLOOR-TO-CEILING FIXED DIVIDER SYSTEM:

Tel. 205.941.1942, info@coronagroupinc.com

B.	Approved Alternates: Due to the unique nature of this product there are no known alternates.

C.	Additional alternates must have prior approval.  Submit technical information and samples to architect a minimum of 14 days prior to bid date

2.2	ACOUSTICOR WEAVE – INTERSECTION FLOOR-TO-CEILING FIXED DIVIDER SYSTEM

A.	INTERSECTION ACOUSTICOR Acoustic Felt Woven Slats

- SLATS:  Woven strips of ACOUSTICOR panels visual privacy and sound absorption – various sizes per drawing

- THICKNESS:  9mm (3/8”) thick

- MATERIAL: 100% Polyester fiber acoustic felt – 60% Recycled PET material and 40% Virgin material

- ECO PROPERTIES: Non-toxic, Non-allergenic, Non-irritant; no coatings/sprays that degrade

- ACOUSTIC RATING: Average NRC .7 (subject to mounting conditions and application)

- FIRE RATING: ASTM E84 CLASS A

- COLORS:  20 Standard Color Offerings

- POSTS AND RAILS:  Satin Anodized Extruded Aluminum, 6105-T5 Alloy with T-slots as designated in drawings

- 1.5” Square Tubes finished on all sides and joined with concealed fasteners

- T-Slots only as needed for mounting and fixing slats, remaining exposed slots shall be trimmed with compatible material

C.	VERTICAL INTERMEDIATE POSTS:  Satin Anodized Solid Aluminum

½” Diameter solid aluminum rods

- Rods to be secured to Rails and notched to lock woven slats in place

2.3		FABRICATION

- Provide factory-assembled unitized Divider Screens when feasible for installation on site using concealed fasteners

- On-Site Assembly:  Provide all Posts, Rails, Rods, and ACOUSTICOR strips to make unitized panels on site for installation using concealed fasteners

2.4		FINISHES

- Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for recommendations relative to applying and designating finishes

B	Manufacturer’s Standard finishes as designated above

C.	Provide full range of color offerings (20 colors) for ACOUSTICOR

PART 3- EXECUTION

3.1		EXAMINATION

A.	Examine FLOOR AND CEILING surfaces, with installer present, for compliance with requirements and other conditions affecting visual display board/track installation

B.	Verify flat floor and ceiling surfaces for proper board installation; provide concealed shims as necessary within Architect’s acceptable variance.

C.	Do not proceed with installation until unsatisfactory conditions have been corrected

3.2		INSTALLATION

- Deliver factory-built divider screens and frames, completely assembled in one piece without joints, where possible

- Install units in locations and mounting heights as indicated on drawings and according to manufacturer’s written instructions.  Keep perimeter lines straight, plumb and level. Provide grounds, clips, brackets, anchors, trim and accessories necessary for complete installation

3.3		CLEANING

A.	Clean units according to manufacturer’s written instructions

B.	Cover surface of all units with protective cover taped to frame

C.	Remove protective covers and tape on date of Substantial Completion

END OF SECTION

Rev. 04-22-2021