- SECTION 10 11 00

- VISUAL DISPLAY BOARDS

- PART 1- GENERAL

- 1.1		SUMMARY

A.	Section Includes:

1.	Dry-erase Frameless Markerboards

2.	Frameless Chalkboards

B.	Related Sections:

1.	Section 06 10 00 – Rough Carpentry: Wood blocking

2.	Section 09 29 00 – Gypsum Board

3.	Section 09 72 00 – Wall Coverings

4.	Section 09 91 00 – Painting

- 1.2		SUBMITTALS

A.	Product Data: Provide technical data for products specified.

B.	Shop Drawings: Provide shop drawings for each type of visual display board specified, including section details indicating trim, face, core and backing materials, wall elevations, dimensions, joint locations between panels exceeding maximum panel length and anchor and installation details

C.	Product Samples: Submit manufacturer’s color chart showing the full range of colors available for the following:

1.	Chalkboards and Markerboards: Actual sections of porcelain enamel steel finish for each type of chalkboard and markerboard required

D.	Contract Closeout Submittals:

1.	Maintenance Data: Manufacturer’s cleaning and maintenance instructions covering both routine (daily or weekly) and long term (yearly or longer) operations

2.	Warranty: Executed copy of manufacturer’s warranty

- 1.3		DELIVERY, STORAGE AND HANDLING

A.	Storage and Protection:

1.	Store factory units horizontally with packaging materials between each unit to prevent damage and warping

2.	Store materials in dry areas at temperatures above 55º F

- 1.4		PROJECT CONDITIONS

A.	Field Measurements: Verify field measurements before fabrication to ensure proper fitting.  Coordinate fabrication schedule with construction progress to avoid delaying the work. Notify the Architect of any conflicts with other construction such as casework, electrical switches, outlets, clocks, mechanical openings, fire detector devices, etc

B.	Environmental Requirements: Install only when the building is enclosed and interior air and substrate temperatures are stable and approximate design conditions

- 1.5		WARRANTY

A.	Porcelain Enamel Writing Surface Warranty: Submit a written warranty executed by the manufacturer agreeing to replace porcelain enamel steel markerboards/chalkboards that do not retain their original writing and erasing qualities, become slick and shiny (if matt surface), or exhibit crazing, cracking or flaking within the specified warranty period, provided the manufacturer’s written instructions for handling, installation, protection and maintenance have been followed

1.	Warranty Period: Forever Warranty on CPS Porcelain markerboards and chalkboards

- PART 2- PRODUCTS

- 2.1		MANUFACTURERS

A.	Basis-of-Design Product: Corona Group Presentation Products – MERGE Frameless Dry-erase Markerboards/Chalkboards

Contact: info@coronagroupinc.com or call Corona Group @ 205.941.1942, coronagroupinc.com

B.	Approved Alternates: (Due to the unique nature of the Frameless Markerboard/Chalkboard system, please contact Corona Group for specific information regarding alternative products)

C.	Additional alternates must have prior approval.  Submit technical information and samples to architect a minimum of 14 days prior to bid date

- 2.2		FRAMELESS MARKERBOARDS

A.	Frameless Markerboard: Magnetic dry-erase markerboards for marker pens and magnets

- Finished Surface: CPS Satin Gloss, scratch-resistant, porcelain enamel steel .4mm -.6mm thick; fused twice between 1200F and 1600F	 to create a non-porous writing surface. Porcelain Steel is Bronze Cradle-to-Cradle Certified and made with 50% post -industrial steel

- Core/ Substrate:

- 1/2”(12mm) Solid KomaCor High-Density Board with Integral Color, laminated to porcelain steel face

- Other thicknesses available based on application

- Sizes: As indicated in Drawings

- Edge Profile:

  - Square Edge for all thicknesses

  - Knife edge in 1/2” (6mm), tapers to a ¼” edge at surface

5.	Writing Surface Color:

a.	Standard Satin Gloss

b.	Optional Colors (Grey and Beige) Extended Lead-Time

6.	Core Substrate Color:

a.	Black or White

b. 	Optional Colors – check availability

7.	Accessories:	Provide one accessory pack for each markerboard.

a.	Accessory Pack for Markerboard shall include:  four (4) or (8) dry-erase markers, one (1) magnetic felt eraser, one (1) package of ten (10) replacement eraser pads, one (1) package of three (3) Eximo™ markerboard cleaning pads and one (1) soft drying cloth, one (1) bottle for water, one (1) magnetic hook, one (1) 12” x12” microfiber towel and twelve (12) round magnets

8.	Optional Penshelf:

a.	Clear anodized aluminum pen shelf – specify length and mounting method:  Adhesive, Magnetic or Wall Mount

b.	Accessory Hub – All-in-One Magnetic Container for All Accessories listed above

- 2.3		FRAMELESS CHALKBOARDS

- Frameless Chalkboard: Magnetic Chalkboards

- Finished Surface: CPS Matt, scratch-resistant, porcelain enamel steel .4mm -.6mm thick; fused twice between 1200ºF and 1600F to create a non-porous writing surface.  Porcelain Steel is Bronze Cradle-to-Cradle Certified and made with 50% post-industrial steel

2.	Core/ Substrate:

- 1/2”(12mm) Solid KomaCor High-Density Board with Integral Color, laminated to porcelain steel face

- Other thicknesses available based on application

3.	Sizes: As indicated in Drawings

4.	Edge Profile:

a. Square Edge for all thicknesses

b. Knife Edge in 1/2” (6mm), tapers to a ¼” edge at surface

5.	Writing Surface Color:

a.	Standard Matte

b.	Optional Colors (Green and Blue) Extended Lead-Time

6.	Core Substrate Color:

a.	Black or White

b. 	Optional Colors – check availability

7.	Accessories:  Provide one (1) accessory pack for each chalkboard.  Accessory Pack for Chalkboard shall include:  one (1) box of ten (10) sticks of white chalk, one (1) non-magnetic foam eraser and twelve (12) round magnets

8.	Optional Chalk Tray:

a.	Clear anodized aluminum chalk tray – specify length and mounting method:  Adhesive, Magnetic or Wall Mount

- 2.5		FABRICATION

A.	Porcelain Enamel steel writing board facing sheet shall be laminated to the core material under pressure with manufacturer’s recommended flexible, waterproof adhesive.

B.	Assembly: Provide factory-assembled writing board units as required per Drawings.  Writing board steel/porcelain edges at joints should be  precision cut and factory sealed

C.	Product is designed as a freestanding Frameless Marker or Chalk Board, typically installed using concealed fasteners

- Boards may be produced to hang in a Vertical or Horizontal Position

- Adjacent boards are intended to align and should be specified and mounted with 1” Aluminum Spline from top to bottom of adjacent boards

- 2.6		FINISHES

A.	Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for recommendations relative to applying and designating finishes

B.     	Manufacturer’s Standard finishes as designated above

- PART 3- EXECUTION

- 3.1		EXAMINATION

A.	Examine wall surfaces, with installer present, for compliance with requirements and other conditions affecting visual display board/rail installation

B.	Verify flat wall surfaces for proper board installation; provide concealed shims as necessary within Architect’s acceptable variance. Minimum Level 4 finished wall required by contractor

C.	Do not proceed with installation until unsatisfactory conditions have been corrected

- 3.2		INSTALLATION

A.	Deliver factory built visual display boards with the minimum number of joints possible

B.	Install units in locations and mounting heights as indicated on drawings and according to manufacturer’s written instructions.  Keep perimeter lines straight, plumb and level

C.	Provide grounds, clips, backing materials, adhesives, brackets, splines, anchors, trim and accessories necessary for complete installation

- 3.3		CLEANING

A.	Clean units according to manufacturer’s written instructions

B.	Cover surface of all units with protective cover taped to frame

C.	Remove protective cover and tape on date of Substantial Completion

- END OF SECTION

Rev. 04-22-2021