SECTION 10 11 00

VISUAL DISPLAY BOARDS

PART 1- GENERAL

1.1		SUMMARY

A.	Section Includes:

1.	Magnetic Dry-erase Glassboards (Unframed)

2.	Magnetic Dry-erase Glassboards (Framed) (Flexible Component Glassboards)

B.	Related Sections:

1.	Section 06 10 00 – Rough Carpentry: Wood blocking

2.	Section 09 29 00 – Gypsum Board

3.	Section 09 72 00 – Wall Coverings

4.	Section 09 91 00 – Painting

1.2		SUBMITTALS

A.	Product Data: Provide technical data for products specified.

B.	Shop Drawings: Provide shop drawings for each type of visual display board specified, including section details indicating trim, face, core, and backing materials, wall elevations, dimensions, joint locations between panels exceeding maximum panel length and anchor and installation details

C.	Product Samples: Submit manufacturer’s color chart showing the full range of colors available for the following:

1.	Glassboard: 12” X 12” Sample to illustrate material, finish, color, and configuration of each type of glassboard required

2	Aluminum Trim and Accessories: Samples of each type and color, on 6-inch (150mm) long sections of extrusions and not less than 4-inch (100mm) squares of sheet or plate. Include sample sets showing the full range of color variations expected

D.	Contract Closeout Submittals:

1.	Maintenance Data: Manufacturer’s cleaning and maintenance instructions covering both routine (daily or weekly) and long term (yearly or longer) operations

2.	Warranty: Executed copy of manufacturer’s warranty

1.3		DELIVERY, STORAGE AND HANDLING

A.	Storage and Protection:

1.	Store factory framed or unframed units vertically with packaging materials between each unit to prevent damage

2.	Store materials in dry areas at temperatures above 55º F

1.4		PROJECT CONDITIONS

A.	Field Measurements: Verify field measurements before fabrication to ensure proper fitting.  Coordinate fabrication schedule with construction progress to avoid delaying the work. Notify the Architect of any conflicts with other construction such as casework, electrical switches, outlets, clocks, mechanical openings, fire detector devices, etc.

B.	Environmental Requirements: Install only when the building is enclosed and interior air and substrate temperatures are stable and approximate design conditions

1.5		WARRANTY

A.	Glass Writing Surface Warranty: Submit a written warranty executed by the manufacturer agreeing to replace glassboards that discolor due to cleaning, staining or use.

1.	Warranty Period: Minimum (2) years from date of completion

PART 2- PRODUCTS

2.1		MANUFACTURERS

A.	Basis-of-Design Product: Dry-erase glassboards are based on Corona Group VIP Series. Contact info@coronagroupinc.com or call Corona Group @ 205-941-1942 – www.coronagroupinc.com

B.	Approved Alternates: (Due to the unique nature of the VIP system, please contact corona for specific information regarding alternative products.)

C.	Additional alternates must have prior approval.  Submit technical information and samples to architect a minimum of 14 days prior to bid date

2.2		MAGNETIC GLASSBOARDS- UNFRAMED

A.	VIP SERIES Magnetic Glassboards: Dry-erase glassboard for marker pens and magnets

1.	Face Sheet: Starphire low-iron, tempered glass with flat polished glass edges

2.	Range:

a.       Thickness: 6mm glass – 1mm galvanized steel plate

b.      Maximum dimensions: 4’ X 9’ (36 sq ft max)

3.	Backing: 1mm galvanized steel plate to produce magnetic properties, as well as safety glass properties

4.	Sizes: As indicated in Drawings

5.	Frame Profile: N/A – Frameless Board

6.	Mounting Hardware: For unframed magnetic glassboards shall be concealed, and of a nature that allow the boards to “float” a maximum of ¼” off the surface of the wall.

7.	Accessories:

a.	Provide one accessory pack for each glassboard.  Accessory Pack shall include four (4) colored pens – red, blue, green and black, one (1) magnetic eraser, with one (1) eraser refill, cleaner, spray bottle and cloth, six (6) heavy duty magnets with protective sleeve

b.	Optional Penshelf:

1.	Acrylic Penshelf or

2.	Penshelf Aluminum – Up to the full width of magnetic glassboard

8.	Board Color: Custom color to be determined and may require custom barrier coat to insure color clarity.

9.	Possible Fabrications: Velour Etch with Opaque Painted Back, Laminated Double Sided with Technographic Interlayer, Laminated with Opaque Metallic Interlayer, Opaque Back Painted with Silk Screen Graphic

2.3		FRAMED GLASSBOARDS

A.	VIP SERIES Framed Glassboards: Magnetic and Non-magnetic Dry-erase glassboards for marker pens and magnets (magnetic)

1.	Face Sheet: Starphire low-iron, tempered glass with flat polished glass edges

2.	Range:

a.       Thickness: 6mm glass – 1mm steel plate

b.      Maximum dimensions: 4’ X 8’

3.	Backing: 1mm galvanized steel plate to produce magnetic, as well as safety glass properties

4.	Sizes: As indicated in Drawings

5.	Frame Profile: Manufacturer’s standard

6.	Top and Bottom Frames: Natural anodized extruded aluminum, 2mm thick, and 60mm wide x 20 mm /40 mm deep.  Rail shall be slotted to accept concealed mounting hardware and ship complete with mounting sets

7.	Sliding Tracks (integral and concealed) for hanging flexible boards; flip charts projection boards, noticeboards, chalkboards, glassboards and additional white boards consistent with VIP Series Design and Specifications

a.	Top frame for Single Rail Glassboards shall have one (1) sliding track

b.	Top frame for Double Rail Glassboards shall have two (2) sliding tracks

c.	Top frame for Double Rail Glassboards shall have a concealed chamber to house ball bearings suitable for displaying paper from the top rail

8.	Side Profiles: Natural anodized extruded aluminum 2mm thick, 6mm wide x 20mm deep

9.	Accessories:

a.	Provide one accessory pack for each glassboard.  Accessory Pack shall include four (4) colored pens-red, blue, green and black, one (1) eraser (magnetic or non-magnetic) with one (1) eraser refill, cleaner, spray bottle and cloth, six (6) heavy duty magnets with protective sleeve (magnetic)

10.	Board Color: Custom color to be determined and may require custom barrier coat to insure color clarity.

11.	Optional:  Full Length Aluminum Pen Shelf docks to bottom rail of glassboard.

2.3	GLASSBOARD – FLEXIBLE COMPONENT BOARDS

A.	VIP SERIES Flexible Glassboard Component Board: Magnetic or Non-Magnetic Dry-erase Glassboards for hanging on VIP single or double top rail

1.	Face Sheet: Starphire low-iron, tempered glass with flat polished glass edges

2.	Range:

a.       Thickness: 6mm glass – 1mm galvanized steel plate

3.	Frame Profile: Manufacturer’s standard

4.	Top and Bottom Frames: Natural anodized extruded aluminum, 2mm thick, and 60mm high x 20 mm deep.  Frame shall be slotted to accept concealed hardware and ship complete with mounting sets for suspension in sliding tracks on single or double rails.

5.	Mounting Hardware: Supplied with standard narrow brackets for the single rail for bi-level presentation systems. Optional deep brackets available for tri-level presentation systems

6.	Factory assembled flexible noticeboards to sizes indicated in drawings or specifications

7.	Side Profiles: Natural anodized extruded aluminum 2mm thick, 6mm wide x 25mm deep

2.4		FABRICATION

A.	Glass Markerboards shall be made using a coating or interlayer applied to an unexposed surface of glass as an opacifier, resulting in a writable surface. A 1mm steel backer shall be laminated to the glass panel to provide magnetic properties. All edges of an unframed glassboard shall be flat polished and cleaned of any overspray.

B.	Assembly: Provide factory-assembled magnetic glass writing board units, as required.  Magnetic glass writing boards shall be delivered in a single panel up to manufacturer’s maximum size to the job site for installation using concealed fasteners. Oversized boards should be installed side by side with 1/8” reveal between each board and installed directly to the wall using concealed fastening systems recommended by the manufacturer.

Joints should be as follows:

1.	Make joints only where total length/height exceeds maximums.  Fabricate minimum number of joints, balanced around center of board, coordinate with Architect for approval	.

2.	Provide manufacturer’s standard vertical butt joint system between abutting sections of writing boards – Standard 1/8” reveal between each board and surrounding millwork for unframed glassboards.

3.	Provide manufacturer’s standard vertical trim at joints between framed glassboards – Maximum 12mm (1/2”) wide H-Profile with finish to match frame.

2.5		FINISHES

A.	Custom color tolerance to be within one delta E (CIE LAB @ D65 10°) (ASTM D 2244)

B.	Comply with NAAMM’s “Metal Finishes Manual for Architectural and Metal Products” for recommendations relative to applying and designating finishes

C.     	Manufacturer’s Standard finishes as designated above

PART 3- EXECUTION

3.1		EXAMINATION

A.	Examine wall surfaces, with installer present, for compliance with requirements and other conditions affecting visual display board/rail installation

B.	Verify flat wall surfaces for proper board installation; provide concealed shims as necessary within Architect’s acceptable variance. Minimum Level 4 finished wall required by contractor.

C.	Do not proceed with installation until unsatisfactory conditions have been corrected

3.2		INSTALLATION

A.	Deliver factory built visual display boards, completely assembled in one piece without joints, where possible.

B.	Install units in locations and mounting heights as indicated on drawings and according to manufacturer’s written instructions.  Keep perimeter lines straight, plumb and level. Provide grounds, clips, backing materials, adhesives, brackets, anchors, trim and accessories necessary for complete installation.

C.	Coordinate project site assembled units with grounds, trim and accessories. Join with a neat, precision fit.

3.3		CLEANING

A.	Clean units according to manufacturer’s written instructions.

B.	Cover surface of all units with protective cover taped to frame.

C.	Remove protective cover and tape on date of Substantial Completion.

END OF SECTION

Rev. 04-22-2021